Spike Planning System

Raspberry Pi Setup, Deployment & Testing Guide

Abhilekh Borah  ·  April 2026

# Overview

I built this system to run a CS:GO-style bomb-planting game mechanic using a Raspberry Pi as the hardware layer and a Mac (or any machine) running the Python backend. The Pi detects USB insert and remove events and publishes them over MQTT. The backend — a FastAPI application — receives those events, runs them through a deterministic state machine, manages timers, and broadcasts state changes back over WebSocket to any connected dashboard.

This document covers everything I need to do to get the Pi wired up, test the full round flow, and understand which files to touch when something needs changing.

# System Architecture

The system is split into two physical nodes:

- Raspberry Pi — detects USB events, publishes MQTT messages, receives GPIO trigger commands

- Backend machine (Mac / server) — runs FastAPI, state machine, timer manager, WebSocket server

Communication between them flows entirely through an MQTT broker running on the backend machine. The Pi never calls the backend HTTP API directly — everything goes through MQTT topics.

The state machine itself is pure Python with no I/O. It only returns lists of events to emit. All side effects (timers, MQTT publishes, WebSocket broadcasts) happen in the surrounding wiring in main.py.

## MQTT Topic Map

Inbound (Pi → Backend):

- pi/events/usb — payload: {"action": "inserted"} or {"action": "removed"}

- pi/events/reset — payload: {} or {"reason": "manual"}

Outbound (Backend → Pi):

- backend/state — payload: {"state": "PLANTING"} — current state after each transition

- pi/output/gpio — payload: {"action": "spike_planted"} — trigger commands for GPIO/audio/lights

## State Transition Table

The full transition table as I defined it in state_machine.py:

- IDLE + USB_INSERTED → PLANTING (starts 60s planting timer)

- IDLE + TIMEOUT → ENDED (round end trigger)

- PLANTING + USB_REMOVED → IDLE (cancels planting timer)

- PLANTING + TIMEOUT → ACTIVE (spike planted, starts 120s spike timer)

- ACTIVE + USB_INSERTED → DEFUSING (starts 60s defusing timer)

- ACTIVE + TIMEOUT → ENDED (spike exploded trigger)

- DEFUSING + USB_REMOVED → ACTIVE (cancels defusing timer)

- DEFUSING + USB_INSERTED → ENDED (defused trigger)

- DEFUSING + TIMEOUT → ENDED (defuse failed trigger)

- ANY + RESET → IDLE (cancels all timers, starts 600s main timer)

# Project Structure

After setting up the subdirectory layout, my Spike_Planning folder looks like this:

Spike_Planning/

├── main.py              ← FastAPI entry point, wiring, HTTP + WS routes

├── config.py            ← Environment variable config (MQTT host/port, log level)

├── requirements.txt     ← Python dependencies

├── dashboard.html       ← Browser-based test dashboard (no build step needed)

├── .env                 ← Optional: override defaults

├── core/

│   ├── __init__.py

│   └── state_machine.py ← Pure state machine, no I/O

├── events/

│   ├── __init__.py

│   ├── event_bus.py     ← Async pub/sub queue

│   └── event_types.py   ← EventType + TimerName enums

├── iobridge/            ← Renamed from 'io' to avoid Python stdlib clash

│   ├── __init__.py

│   ├── mqtt_client.py   ← MQTT ↔ EventBus bridge

│   └── websocket_manager.py

└── timers/

├── __init__.py

└── timer_manager.py ← asyncio timer management

I renamed the io/ folder to iobridge/ because 'io' is a reserved name in Python's standard library. The import in main.py reads: from iobridge.mqtt_client import MQTTClient

# Backend Setup (Mac / Server)

## 1. Install dependencies

cd Spike_Planning

pip install -r requirements.txt

## 2. Install and start Mosquitto (MQTT broker)

I use Homebrew on Mac to run the broker locally:

brew install mosquitto

brew services start mosquitto

To verify it is running:

brew services list | grep mosquitto

## 3. Create a .env file (optional)

By default, config.py connects to localhost:1883. To override, I create a .env file in Spike_Planning/:

MQTT_HOST=0.0.0.0

MQTT_PORT=1883

LOG_LEVEL=INFO

## 4. Add CORS middleware to main.py

Since the dashboard.html opens directly from the filesystem (not a dev server), I need to allow cross-origin requests. I added the following to main.py:

At the top with other imports:

from fastapi.middleware.cors import CORSMiddleware

Right after the app = FastAPI(...) line:

app.add_middleware(

CORSMiddleware,

allow_origins=["*"],

allow_methods=["*"],

allow_headers=["*"],

)

allow_origins=["*"] is fine for local testing. Before deploying to any production environment I should lock this down to specific origins.

## 5. Start the backend

python main.py

The server starts on port 8000. On startup it emits a RESET event, putting the state machine into IDLE and starting the 10-minute main timer. MQTT connection errors in the logs are expected until the Pi connects — they do not affect state machine operation.

# Raspberry Pi Setup

## Prerequisites

- Raspberry Pi (any model with WiFi) running Raspberry Pi OS

- Both the Pi and the Mac on the same local WiFi network

- Python 3.9+ on the Pi

## 1. Find the Mac's local IP address

On the Mac, I run:

ipconfig getifaddr en0

This gives me something like 192.168.1.10. I use this IP in all Pi-side configuration.

## 2. Install paho-mqtt on the Pi

pip install paho-mqtt

## 3. Create the Pi-side event sender script

I create a file called pi_sender.py on the Pi. This script listens for actual USB device events from the OS and publishes them to the MQTT broker on my Mac:

import paho.mqtt.client as mqtt

import json, subprocess, time

BROKER_IP = "192.168.1.10"  # Replace with Mac's actual IP

BROKER_PORT = 1883

client = mqtt.Client()

client.connect(BROKER_IP, BROKER_PORT)

client.loop_start()

def on_usb_inserted():

client.publish("pi/events/usb", json.dumps({"action": "inserted"}))

print("Published: USB inserted")

def on_usb_removed():

client.publish("pi/events/usb", json.dumps({"action": "removed"}))

print("Published: USB removed")

def on_reset():

client.publish("pi/events/reset", json.dumps({"reason": "manual"}))

print("Published: Reset")

For actual hardware USB detection, I can monitor udev events:

# Run this in terminal to see USB device events in real time

udevadm monitor --subsystem-match=usb --property

For a production build I would write a udev rule that calls a script on USB insert/remove, which then publishes to MQTT. For testing, manually calling on_usb_inserted() and on_usb_removed() from a Python REPL is sufficient.

## 4. Create a manual test script for the Pi

For initial testing without real USB hardware events, I use this script to simulate events from the Pi:

# test_pi.py — run on the Pi

import paho.mqtt.client as mqtt

import json, time

client = mqtt.Client()

client.connect("192.168.1.10", 1883)

# Simulate a full round

time.sleep(1)

client.publish("pi/events/usb", json.dumps({"action": "inserted"}))  # IDLE → PLANTING

time.sleep(3)

client.publish("pi/events/usb", json.dumps({"action": "removed"}))   # PLANTING → IDLE

time.sleep(2)

client.publish("pi/events/usb", json.dumps({"action": "inserted"}))  # IDLE → PLANTING again

time.sleep(62)  # Let planting timer expire → ACTIVE

client.publish("pi/events/usb", json.dumps({"action": "inserted"}))  # ACTIVE → DEFUSING

time.sleep(3)

client.publish("pi/events/usb", json.dumps({"action": "inserted"}))  # DEFUSING → ENDED (defused)

python test_pi.py

## 5. Receive GPIO commands on the Pi

The backend publishes to pi/output/gpio whenever a game event occurs (spike planted, defused, exploded, etc.). I subscribe to this topic on the Pi to trigger lights, audio, or GPIO pins:

# gpio_listener.py — run on the Pi

import paho.mqtt.client as mqtt, json

def on_message(client, userdata, msg):

data = json.loads(msg.payload.decode())

action = data.get("action")

print(f"GPIO trigger: {action}")

if action == "spike_planted":

pass  # trigger LED / buzzer

elif action == "defused":

pass  # trigger win sound

elif action == "spike_exploded":

pass  # trigger explosion effect

client = mqtt.Client()

client.on_message = on_message

client.connect("192.168.1.10", 1883)

client.subscribe("pi/output/gpio")

client.loop_forever()

# Testing With the Dashboard

## Opening the dashboard

I open dashboard.html directly in any browser on the Mac — no web server required:

open Spike_Planning/dashboard.html

On the Pi, I open it in Chromium and update the two constants at the top of the script block inside dashboard.html:

const BASE = 'http://192.168.1.10:8000';

const WS   = 'ws://192.168.1.10:8000/ws';

## Full round test sequence

With the dashboard open and main.py running, I step through the full game flow using the inject buttons:

- State shows IDLE, main 10:00 timer is counting down

- Click USB inserted → state changes to PLANTING, 1:00 planting timer starts

- Click USB removed → state returns to IDLE, planting timer cancelled

- Click USB inserted again → PLANTING, timer restarts

- Click Timeout — planting → ACTIVE, 2:00 spike timer starts, spike_planted trigger fires

- Click USB inserted → DEFUSING, 1:00 defusing timer starts

- Click USB inserted again → ENDED, defused trigger fires

- Click Reset → back to IDLE from any state

I can also let timers run naturally to test timeout transitions. The dashboard shows a live countdown and turns the timer red at 10 seconds remaining.

## Monitoring MQTT traffic

To see every message flowing through the broker in real time, I run this on the Mac:

mosquitto_sub -h localhost -t "#" -v

This is the fastest way to confirm that Pi-side publishes are reaching the broker and that the backend is publishing GPIO commands back correctly.

# File Change Reference

This section documents exactly which file to edit for each type of change.

## config.py — Change MQTT host or port

I update the defaults here, or override with a .env file:

mqtt_host: str = "localhost"   # change to Pi's IP if broker runs on Pi

mqtt_port: int = 1883

## state_machine.py — Change game timers or transitions

All timer durations are defined in the transition handler methods. To change the planting window from 60s to 90s:

# In _idle_usb_inserted()

EmittedEvent(EventType.START_TIMER, {"name": TimerName.PLANTING, "duration": 90})

To add a new transition, I add a handler method and register it in _TRANSITION_TABLE:

(State.IDLE, EventType.RESET): _any_reset,  # existing

(State.ACTIVE, EventType.MY_NEW_EVENT): _my_new_handler,  # new

## event_types.py — Add a new event or timer

All event names and timer names live here as string enums. Adding a new event:

class EventType(str, Enum):

USB_INSERTED = "USB_INSERTED"

MY_NEW_EVENT = "MY_NEW_EVENT"   # add here

## iobridge/mqtt_client.py — Add a new MQTT topic

To handle a new inbound topic from the Pi, I add a branch in _handle_message():

elif topic == "pi/events/my_new_topic":

await self._bus.emit(EventType.MY_NEW_EVENT, data)

To publish a new outbound topic, I add a handler method and subscribe it in main.py:

async def on_my_event(self, _event_type, data):

await self._publish("pi/output/my_topic", data)

## main.py — Wire a new subscription

Any new event handler needs to be registered in wire_subscriptions():

bus.subscribe(EventType.MY_NEW_EVENT, my_module.on_my_event)

# Troubleshooting

## MQTT connection errors in logs

These appear on startup if no broker is running. They repeat every 5 seconds but do not crash the backend. I start Mosquitto with brew services start mosquitto on the Mac to resolve this.

## Dashboard shows Load failed errors

This is a CORS issue. I make sure the CORSMiddleware block is in main.py and that the server has been restarted after adding it.

## Pi cannot reach the broker

I verify both devices are on the same WiFi network, confirm the Mac's IP with ipconfig getifaddr en0, and check that port 1883 is not blocked by a firewall. On Mac I can temporarily allow the connection with:

sudo /usr/libexec/ApplicationFirewall/socketfilterfw --add /opt/homebrew/sbin/mosquitto

## State machine ignores an event

The state machine logs a warning for any event that has no handler in the current state — Ignored event X in state Y. This is intentional. I check the transition table in state_machine.py to confirm the (state, event) pair is registered.

## Import error: No module named 'io.mqtt_client'

Python's standard library has a built-in module called io. I renamed my folder to iobridge/ and updated the two imports in main.py accordingly:

from iobridge.mqtt_client import MQTTClient

from iobridge.websocket_manager import WebSocketManager

Abhilekh Borah  ·  Spike Planning System  ·  April 2026